Name of Bidder (Registered company name): ____________________________________________________

| # | Descriptions | Unit | Qty | Price I D | Total Cost ID |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavations in all types of soil and rocky area for PE pipes with width 0.6 m, depth 1m in all types of soils , the work includes excavation in asphalt layers, removing of any access material, clearing the site entirely and according to the instruction of site Engineer. | Meter. length | 3,500 |  |  |
| 2 | Supply and Install polyethylene pipe (PE 100 SDR 11 PN 16 BAR), BLACK for drinking water OD 160 mm, the price include provide all the required machines, materials, manpower, etc…. for connecting the pipes by using mechanical fittings, the work includes supplying the all required fittings like check valve, tees, reducers...etc. and according to the instructions of the supervising engineer. | Meter. length | 3,500 |  |  |
| 3 | Backfilling work for the pipes lines using sand around the pipe by 25 cm thick, then filling by pure soil. | Meter. length | 3,500 |  |  |
| 4 | Manhole: Supply materials to construct Reinforced concrete manhole class 1:2:4 with dimension (1x1) m the price include supply and install a ductile 6" vent valve, shuttering, and metal cover 2mm thickness with iron angle frame 1.5” according to the drawing. | No. | 3 |  |  |
| 5 | Manhole for beginning and ending points: Supply materials and construct Reinforced concrete manhole class 1:2:4 with dimension (1.5x1.5) m the price include shuttering and metal cover 2mm thickness with iron angle frame 1.5” according to the drawing. | No. | 2 |  |  |
| 6 | Supply and install a ductile 6" valve for main source the price including cutting pipe, welding, re-connecting, excavation, backfilling…etc. | No. | 2 |  |  |
| 7 | Supply and install a ductile 4" valve for water network the price including cutting pipe, welding, re-connecting, excavation, backfilling…etc. | No. | 3 |  |  |
| Total Cost in IQD |  |  |  |  |  |

Delivery Lead-Time:  _____________________________ days to DRC IRAQ

Delivery Lead-time is defined as the time from receipt of a DRC Purchase Order to the delivery of the Goods to the DRC

- Bid currency must be in Iraqi Dinar (IQD). No other currency is acceptable

- We unconditionally state that if awarded the Contract the delivered Service will be in accordance with the specifications listed above, including the required size.

Name of Bidder (Registered company name): _____________________________________________

Required Delivery Destination: Zummar, Ninewa Governorate.

This Bid is valid for: 30 days

Currency of Bid: IQD (no other currency is acceptable)

| Company Name: __________________________________ | Contact person: ___________________________________ |
| --- | --- |
| Address: ________________________________________ | Phone No: __________________ |
| ________________________________________ | Email Address: __________________________________ |
| Signed: ________________________________________ | Title/Position: ___________________________________ |
| Print Name: ____________________________________ | Date: _______________________ |
| A duly authorized company representative. | Please stamp this Bid Form with your Company Stamp |