A. TECHNICAL SPECIFICATIONS OF Calcium hypochlorite (Chlorine Powder)

|  | Item Description |  | Unit | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Calcium hypochlorite (Chlorine Powder) Calcium hypo chloride 65-70% concentration in plastic container double packing capacity (40-50) kg with 1kg measuring cu. expire date not less than 2 years from the date of producing printed on the container with the name of company. Terms and Conditions: presentation of original product certificate. Synonym(s): Calcium oxychloride, bleaching powder, Chlorinated lime and granulated chlorine Chemical Family: Salt of hyprochlorous acid Molecular Formula: Ca(OCI)2 PIN – UN/NA Number(s): UN1748, UN2880 Product Use: Disinfection in swimming pools and drinking water supplies; treatment of industrial cooling water; clime control; odor control; sewage and waste water treatment HAZARDOUS INGREDIENTS Ingredients: (by weight) 65% Calcium hypochlorite – Inerts 35% 70% Calcium hypochlorite – Inerts 30% CAS Registry Number – Calcium hypochlorite: 7778-54-3 CAS Registry Number – Calcium hydroxide: 1305-62-0 PHYSICAL DATA Physical State: Solid Odor and Appearance: White, free flowing granular solid, with a strong chlorine odor Odor Threshold: Not available Specific Gravity: 2.00-2.20 @ 20°C (water=1@40°C) Vapor Pressure: Not applicable Vapor Density: Not applicable Evaporation Rate: Not applicable Boiling Point: Not applicable Melting Point: Decomposes at temperatures above 100°C pH: 11.5 (5% solution) Coefficient of Water/Oil Distribution: Not applicable Route of Entry and Effects of Short Term (Acute) Exposure – Inhalation: dust and mist irritate the nose and throat. When mixed with acids, chlorine gas is released. This gas causes irritation of the respiratory tract. Prolonged exposure to thigh concentrations of chlorine gas may result in severe lung damage. Eye Contact: Exposure to calcium hypochlorite can cause eye damage. Concentrated solutions cause burns which may result in permanent eye damage if not promptly treated. Skin Contact: Calcium hypochlorite dust and solutions can cause irritation, and in severe cases, chemical burns. Ingestion: When ingested, there will be burning in the mouth and throat. Calcium hypochlorite can cause abdominal cramps and nausea, which may lead to convulsions, coma and death. Route of Entry and Effects of Short Term (Acute) Exposure – Skin irritation may occur from repeated or prolonged skin contact. Exposure Limits – Time Weighted Average (TLV-RWA): Not available Irritancy: Not available Sensitization of Product: Not available Animal Toxicity Data – AC50: (Inhalation, rats) – 148 mg/l LD50: (Oral, rats) – 1300 mg/kg Carcinogenicity: Not carcinogenic (IARC and ACGIH) Reproductive Toxicity: Not available Name of Toxicologically Synergistic Products: Not available. |  | Ton | 15 |
|  | Calcium Hypochlorite 65% - 70% Molecular Formula: Ca(OCI)2: |  |  |  |
|  | Appearance | White or light-grey granular |  |  |
|  | Available chlorine | 70%min |  |  |
|  | Moisture | 5.5-10% |  |  |
|  | Insolubility | 3.0 %max |  |  |
|  | Density g/ml | 1.0-1.1 |  |  |
|  | CaCO3 | 3%max |  |  |
|  | Ca(OH)2 | 2%max |  |  |
|  | CaCL2 | 2%max |  |  |
|  | PH (5% SOLUTION) | 12-Nov |  |  |
|  | MgCL2 | 0.01%max |  |  |
|  | Fe2O3 | 0.001%max |  |  |
|  | Al2o3 | 0.0001%max |  |  |
|  | For 1ppm | For 1000lt Use 1.5-1.7g |  |  |
|  | Appearance | White or light-grey granular |  |  |

B. Remarks and Conditions

| 1 | Standard of products should be compliant with (Turkish and European) standards, with good quality, all the product should be received and checked by related engineers for approval. And product must be succeeded in one of Kurdistan university lab' |
| --- | --- |
| 2 | All items should be Delivered to directorate of water Duhok warehouse in Etit |