RFQ Title: Shelter Kits

RFQ Submission Deadline: 15th November 2018, 16:00/Human Relief Foundation, Dream City Villa 29, Erbil, Iraq

RFQ Ref#: HRFRFQSHKT01080518

Summary:

Vendors are invited to submit a quotation for 32,250 shelter kits, to contain the following items:

| Item | Minimum Specifications | Unit | Quantity |
| --- | --- | --- | --- |
| Steel timber nails | 65 mm Size: 8d Small Galvanized Nail 100% HDG/ hot dip galvanized steel in accordance with ASTM-A153 Headed nail Approximate quantity per kilo: 200 - 250 | kg | 1 |
| Clear plastic sheets | 190 GSM (gram per square meters) Dimensions: 6*4m | piece | 2 |
| Expanding spray foam | Polyurethane Multipurpose Foam Application temperature from +4°C to +45°C Expansion capacity 50% of original volume or higher Closed cells, R-value 6 or higher. Tack Free Time 5-15min at 21C Curing Time 50-100 min Cutting Time 50-100 min Density 18-25 kg/m3 Fire Behaviour: Class B3 Joint sound insulation 58dB Temperature Resistance From -40°C - +90°C Shelf life of 15 months after delivery In accordance with FEICA minimum standard | tube | 4 |
| Nails with washer head | 2.70mm × 70mm Nails with washer head / roofing nail 100% HDG/ hot dip galvanized steel in accordance with ASTM-A153 Washer in rubber with ardox body 10d / 18x30 (JPxLPP) / 2 ¾ x 10 (Inch x BWG) Approximate quantity per kilo: 50 | kg | 1 |
| Binding wire | Roll of binding wire, 1.8-2.5mm thickness; 30m long | roll | 1 |
| Galvanized concrete nails | 4.00 mm x 80mm Size: 12d High quality carbon steel, zinc plated With smooth shank and round cap | kg | 0.5 |
| Duct tape | Polyethylene film over cloth scrim and rubber adhesive Length 54.8m – Width 48mm Adhesion to Steel: 62~72 oz./in. width (79 N/100 mm) Ref: D-3330 Tensile Strength: 19 ~32 lbs./in. width (560 N/100 mm) Ref: D-3759 Elongation at Break: 15% ~ 24% Ref: D-3759 Thickness: 5.8~8.1 mils (0.147-0.21 mm) Ref: D-3652 Temperature Use Range: Up to 200°F (93°C) | roll | 1 |
| Saw | Carpenter all-purpose – standard coarse cut saw 2D/3D teeth type Blade in forged steel, BD2/BS4659 Handle in hardwood Blade material: spring steel Blade size: 20" (508mm) Blade thickness: 1mm 20"""" Standard Coarse Cut Saw Total length: 750mm Teeth per Inch: 7~9 pt. | piece | 1 |
| Utility knife | Folding Retractable Utility Knife Blade Length 100~150mm Handle Length 150mm Color: dark color Number of blades: minimum 3 Handle in rubber covered metal or hard plastic Style: Retractable Weight: 0.15~0.25kg | piece | 1 |
| Caulk gun | 10 oz metal ratcheting caulk gun | piece | 1 |
| Silicone caulking tube | 100% clear, 10 oz tube | tube | 2 |
| L-shaped galvanised steel angle | 42x42x40mm Thickness 2 mm | piece | 4 |
| Flat L-corner | Galvanized Steel 50mm x 50mm / 5cm x 10cm metal plates with screws | piece | 4 |
| Metal handle for door | Zinc or Aluminium, with screws (for wood) 10 to 15 cm length | piece | 2 |
| Hinges with screws | For door installation | piece | 4 |
| Latch | Galvanised steel: 6mm diam and 5-10 cm long | piece | 2 |
| Open shackle padlock | 60mm, with keys | piece | 2 |
| Exterior grade plywood | Size: 12mm thick x 1.21m x 2.42m OSB/3 – Load bearing board for humid conditions Density (ρ): 450-500 (kg/m3) Bending strength - major axis (EN 310) : 20N/mm3 Bending strength - minor axis (EN 310) : 10N/mm3 Modulus of elasticity in bending - major axis (EN 310) : 3500N/mm3 Modulus of elasticity in bending - minor axis (EN 310) : 1400N/mm3 Internal bond (EN 319) : 0.32N/mm3 Swelling in thickness in 24h (EN 317) : 15% Dimensional change at 1% change in panel moisture content: 0.02 (Length %), 0.03 (Width %), 0.05 (Thickness %) Approximate Moisture Content (20oC/85% humidity): 15%”” | piece | 4 |
| Torch | 3 LED Dynamo Wind Up Flashlight Hand Press Crank NR No Battery Torch Light Specifications: Color:Yellow/Red/Blue Material: Plastic Size:10×5×2.5cm | piece | 1 |
| Screwdriver | Drive type: crosshead/flathead- in-line with screws provided Length: 150mm Handle: plastic or wooden | piece | 1 |
| Claw hammer | Weight: min 750g Handle: wooden | piece | 1 |
| Square cut timber | 2cm x 6cm x 3m, rough cut, kiln dried lumber, | piece | 9 |
| Square cut timber | 2.5 cm x 10cm x 300 cm, rough cut, kiln dried lumber, | piece | 4 |
| Bag | CANVAS Tool bag, water resistance, | piece | 1 |

- HRF reserves the right to alter and amend the size and quantity of items listed above.

- HRF reserves the right to accept bids in part or in full.

Submission Requirements:

- Detailed BoQ - stamped

- Detailed pricing, and price breakdown - stamped

- Printed BoQ and pricing - no handwriting

- Completed itemised spreadsheet.

- No correction pen on any document

- Proposals in sealed envelopes

- Place a CD containing soft copies of the documents in the envelope

- Put the RFQ title, ref# and vendor name on the envelope and CD

- Put the RFQ ref# and name on each page of your quotation

- No email submission

- Quotation currency: United States Dollars.

- If the submission deadline falls on a weekend or holiday, the next working day will be the new deadline

- Interested vendors should submit all the required documents to HRF's Erbil Office. Address: Human Relief Foundation, Dream City Villa 29, Erbil, Iraq.

Payment:

- All HRF payments are made within 30 days of correct invoice submission

- Correct invoices are submitted only after satisfactory completion of the work and GRN signing

- Payment methods: bank transfer or cheque (bank transfer is preferred)

- No cash payment under any circumstances

- The vendor is responsible for any bank charges from their end

- The vendor is responsible to provide a bank account that works with HRF’s bank in Iraq

Vendor Registration:

- Unregistered vendors are required to complete their registration before the end of the submission deadline

- Vendors that fail to register on time will not be considered in the opening session

- To complete vendor registration, please see the attached Vendor Registration form in this advertisement

Samples:

- HRF may request samples of proposed items for testing, samples won’t be returned to the vendors unless stated otherwise by HRF in writing

- Samples must be clearly marked to indicate the RFQ and vendor name

- A list of samples and their serials, if available, to be submitted with the samples

- All samples must be placed in appropriate boxes and delivered to HRF office as per instructions

- Sample requests are made prior to award

- If any lab test is required, the vendor will be liable for the test cost

Other:

- Bonds: not required at the RFQ stage

- Site visits: not required

- Pre-proposal meeting: not available

- Technical data: required where applicable