LOT 1 – Workshop Tools

| No. | Item name | Description | Quantity | Reference Picture |
| --- | --- | --- | --- | --- |
| 1 | Workshop Tool Trolley عربة ادوات الورشة | - Empty trolley dimensions Approx. (1000h x 1070w x 460d mm) - Drawers: min. 10 drawers - Material: Metal - Rigid design with work top. - Side plates with holes to attach accessories. - Rubber corners for impact protection. - Heavy duty transport for comfortable steering content label on each drawer or similar product. | 24 |  |
| 2 | Portable Engine Stands حامل المحرك | - Approx. Dimensions: 90*40*17 - Material: Steel - Movable engine stands with a net weight of 40-60 Kg - Capable of lifting 900-1100Kg smoothly and safely with gearbox - Universal Fit, 4-point Universal Mounting Brackets that Fits Most Engines - Practicality, 360º Swiveling & Locking Wheels for Easy Maneuverability and Extra Stability | 12 |  |
| 3 | Air Compressor ضاغط الهواء | - Discharge / Operating Pressure: min. 10 bar - Dimension Approx.: 1600*575*1090MM - Horse Power: min 10hp - Min. 350 lt. capacity - Belt driven single stage twin cylinder compressor. The compressor has two quick – action couplings. One coupling for regulated pressure and one for full, unregulated tank pressure | 12 |  |
| 4 | Battery Charger Station (Middle) "محطة شاحن البطارية الوسطية" | - Input Voltage 220V. - Instant electric current 1250A -1450 A Force start, slow and fast charge 12-24 v - Protection device for over loading and short circuit. Display for voltage and current. Unique automatic monitor for voltage and current. Time regulation system with high accuracy. - Double - layer bridge communication. With fast charge & slow charge (12/24). (HUA) curve silicon steel chip 100% copper core transformer. | 12 |  |
| 5 | Brake Fluid Tester Pen قلم فحص سائل الفرامل | - Auto Power off, 1 LR03 AAA Battery - Size: 15-19cm Length (Approx.) - Material: Plastic - Weight: Approx. 150-250g | 12 |  |
| 6 | Parallel Vice Swivel Large ملزمة رابطة ذات قطب متوازي كبير (منكنة) | - Bench Vice. - All steel Fixed / Swivel Base Casted Iron Painted Body approx. 6inch - Jaw Opening: approx. 6 inch Jaw Width: 6”. - Approx. Dimensions: 40 x 23 x 20.6 cm | 12 |  |
| 7 | Air Impact Wrench مفتاح براغي انج | - Screwdriver: approx.1 inch - Bolt Capacity: approx. 45mm - Max. Torque min. 1800ft.lb (2,445Nm) - Net Weight approx.: 10.61kg - Length approx. :537mm - Air cons approx.: 0.85m3 /min - Air Inlet :1/2 inch - Air Pressure approx. :90-115 Psi - Sound Pressure: max. 98dBA Vibration: max. 9.6m/S 2 | 12 |  |
| 8 | Hand Pallet Jack Wide Forks جك يدوي نقال | - Approx.2 Ton load capacity - Wide forks standard with fork width of approx. 600- 685 mm and length of 100-1.22m | 24 |  |
| 9 | Manual Hydraulic Press منصة الضاغط الهيدروليكي اليدوي | - 10 ton Stand - Material: Iron - Height approx.: 2-2.5m - Width approx.: 1.2-1.6m - Movable piston, - overload safety valve, double speed control unit, pressure gauge, - manual up/down control unit 10-20 ton | 12 |  |
| 10 | Pneumatic Axle Jack جاك هوائي محور العجلة | - Two stage pneumatic jack with approx. 100-111mm height at 80ton +93mm at 50 ton and min height of 255mm | 12 |  |
| 11 | Axle Stand حامل محوري | - Min. 20 Ton Heavy Duty jack stand with max height of 1.05M and minimum height of 675mm - Dimensions: 12 x 12 x 14.5 inches | 24 |  |
| 12 | Crawler Board عربة او نقال زاحفة | - Capacity approx.: 140 -200 Kg. - Weight approx.: 7-11 Kg - Color approx.: Orange or Red - Dimension approx.: 40-42” L 15-17” W & 4”H - Caster wheels: min. Six Rotating Caster wheels with 3-5” Inch - Padded seat & Headers for ultimate comfortable - Practical gap for the tool’s storage Easy cleaning | 48 |  |
| 13 | Stand Drill دريل ثابت ملم | - Drill stand cast iron base, Size approx.: ᴓ43mm, - Volts approx. / hz 230 - 240 volt / 50 hz 1000W - Hight approx.: 1800-2000mm - Depth adjustment scale for drilling depth 150-200mm - Three speeds with the safety switch off | 12 |  |
| 14 | Grinder كوسرة مع حمالة واط | - Approx. Volts / hz 230 - 240 volt / 50 hz RPM 2000-2800 - wattage 550W -1000W - Bench top grinding machine complete with 200mm grinding - wheel grade 60 and grade 30 - preferable with bench grinder stand | 12 |  |

LOT 2 – Safety Equipment’s

| No. | Item name | Description | Quantity | Reference Picture |
| --- | --- | --- | --- | --- |
| 1 | First Aid Kits حقيبة اسعافات اولية | First aid, middle size This kit contains: Scissor 1 Pc Dettol Liquid 1 Pc Paracel 500 Tab 1 Pc Nicip Tab 1 Pc Cotton 15 gm 1 Pc Micropore 1 Inch Betadine Point 15g Burnell Cream 10gm Lop amide Tab 1 Pc Soft swab 10 cmx 10 cm Bandage (3m/10cm) 1 Pc, with all necessary medical needs. | 24 |  |
| 2 | Fuel fire extinguisher type مطفأة حريق امكافحة حرائق الوقود | Are fires involving flammable liquids such as fuel, size 6-12 liter with expiry date more than 1 year. The origin must be similar to EU standards or other equivalent standards. | 24 |  |
| 3 | Electrical fires extinguisher type مطفأة حريق امكافحة حرائق الكهرباء | Are fires caused by electricity, size 6-12 liter with expiry date more than 1 year. The origin must be similar to EU standards or other equivalent standards. | 24 |  |

LOT 3  – Truck Tire Equipment’

| No. | Item name | Description | Quantity | Reference Picture |
| --- | --- | --- | --- | --- |
| 1 | Truck Tire Changer مغير اطار الشاحنة | Portable console, for wheel with a max Diameter of 1.5M wheel width up to 1M, 8.5 thickness rein forced plates on the lifting arm 130-150 bar, Rim diameter 14 inch to 26 inches. | 12 |  |
| 2 | Truck Wheel Balance موازن عجلة الشاحنة | Tier lifter. Have 3 kinds of ALU mode and STA mode, two different function for cars and trucks 1.2M max width of 512mm Power Supply: 220V Wheel weight: 100-160kg Imbalance Resolution: ±1g/±10g Working ambient: 5°C-40°C Noise: <70db. | 12 |  |

Remarks

- Bidder can apply for all LOTS or One LOT individually.

- Minimum 1 years warranty period for all goods are required. Please specify.

- All reference pictures are not an indication for any specific brand but only to understand the description.

- All dimensions and physical characteristics are approximate not exact.

- Supplier must submit Technical Data Sheet / catalogue or brochures of offered product (Please specify the exact model).

- DAP, Baghdad (Iraq).