Section II: Schedule of Requirements

eSourcing reference: RFQ/2019/12557

Technical specifications for Goods and Comparative Data Table:

| Item No | UNOPS minimum technical requirements | Quantity | Is quotation compliant? Bidder to complete | Details of goods offered. Bidder to complete |
| --- | --- | --- | --- | --- |
|  | 1) Manufacturer's Name 2) Country of Origin 3) Brand /Model No 4) Year of Manufacture |  | ☐ Yes ☐ No | Please provide the details of offered product |
| 1 | 25kva Generator (3 phase) set including the diesel engine, alternator, control panel, automatic starting equipment, fuel tank with a built-in canopy suitable for outdoor installation | 1 | ☐ Yes ☐ No | Insert details (if required) |
| 1.1 | Engine Fuel Type: Diesel Cooling: Water-cooled diesel Engine, 50˚C radiator cooled. Governor: Electric Starting system: 24 Volt DC motor, lead acid battery with charging system. Speed: 1500 RPM, direct injection No. of Strokes: 4 |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.2 | Output / Performance Rating: 25KVA (Prime) Voltage: 220-415 Volts Frequency: 50 Hz ( AVR ) : Automatic Voltage Regulator |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.3 | Wiring & connectivity: Connectivity of generator with main DB at the distance of 100 meter using suitable size of cable. |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.4 | Warranty: One year from the date of start up •Attend any minor service/repair/ breakdown within 6 hours upon request. |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.5 | Canopy: The generator set mounted in a sound attenuated and weather protective canopy, |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.6 | Availability of spare parts and after sale service in Baghdad / Iraq |  | ☐ Yes ☐ No | Insert details (if required) |
| 1.7 | Installation Fees |  |  |  |
| 2 | ATS Panel: Complete ATS (Automatic Transfer Switchboard (0-300 amps) | 1 | ☐ Yes ☐ No | Insert details (if required) |

Delivery requirements and Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | Bidder shall deliver the goods Two (2) weeks after Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP, Iraq, Baghdad UNMAS Office in Green Zone | ☐ Yes ☐ No | Insert details |
| Consignee details | Hanjuin Keka +964750 490 3415 Hanjuink@unops.org | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- 20% , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details |

00ax: +45 45 33 75 01