- TECHNICAL SPECIFICATIONS

| Lot 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | Description | Quantity | Reference Picture | Comparative Specification Comparative specification / remarks should indicate any deviation from technical parameter or functional description of tender specification or confirmation with "Yes" and if applicable further particularities. |
| - Electronic Supplies |  |  |  |  |  |
| 1.1 | Supply and extend wires | (2x2.5) mm² length (600 M.L). To connect electricity for interior switches with all requirements to cover all wires using PVC trunk wire size (25 mm). 2-4 wires in each PVC trunk | 600ml |  |  |
| 1.2 | Plug Socket Switches | Supply and fix 13 Ampere , for one-stop-shop, type of switch contains Neon Light. Price includes all requirements for fitting all electricity works at One-stop-Shop counter. | 20pcs |  |  |
| 1. 3 | Supply and fix 10 Ampere Switch | 3 gangs two-way switch for fluorescent strip, work includes all requirements for fixing. Switches | 6pcs |  |  |
| 2. | Air Cooler | - Water tank capacity: 20 liters - Blower function - Rigid plastic body - Cooling module 20000 m²/hr - 1 channel - 3-speed control - Honeycomb cooling pad - Air swingable for both sides, vertical and horizontal - With an original remote control - Movable and with 4 tires - Proper pipe to be installed in order for the water reaches the cooling pads | 2pcs |  |  |
| 3 | Wall mount fan | - Motor: 14mm motor (motor size can be adjustable) - Blade: 3 PP blade - Voltage: 220V/50Hz - Plug: Two round pin plug - Color: Can be grey or black - Body material type: PVC | 6pcs |  |  |
| 4 | STORAGE WATER COOLERS | - Capacity (40 LTR/HR) - Storage capacity (80 LTR) - Of faucets 2 - Dimensions (WXDXH) (665x 485 x 1210) mm (plus or minus 20 mm of the Dimensions are acceptable) - Weight 45 Kg (plus or minus 2KG are acceptable) - Refrigerant R22 /R 134A - Current amps are 2 - Type of compressor ROTARY / RECIP - Silver or White colour - Work includes all requirement to install water pipes, electricity sockets, draining | 1pcs |  |  |

| Lot 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Item | Description | Quantity | Reference Picture |  |
| 1 | Wooden desks | - Classic type. Material made of lamination wood type HDF (High density fibres) - Paper & veneer - High gloss finish - Colours brown or grey or red. - Dimensions (LXWXH) mm (1400 X700 X740) mm - Three drawers’ size (5000 x4000 x5500) mm - Drawers with Key to be fixed to the desk - Hole/cord for computer cable and wires | 6pcs |  |  |
| 2 | Office chair | - Moveable Leather office chair. - Dimensions: 24.75"W x 27.25" D x 38-40.5"H. - Sit on smooth rolling multi- Surface dual wheel Casters. - Chrome base to provide easy Access to your surrounding work area. - Heavy-duty chair supports Up to 300 lbs. - Arm height adjustability allows Precise arm Positioning and added hand (arms rest). - Wrist and forearm comfort. - Black color. | 6pcs |  |  |
| 3 | Fixed chair | - Wood arm rest arm style. - Seat &back Material leather. - Seat Color black /brown. - Weight Capacity approx. 225 Ib. - Chair frame steel. - Seat Width 450 mm. - Seat depth 480 mm. - Height support 820 mm. - Cushion Thickness 3". | 30 pcs |  |  |
| 4 | Fire extinguisher - Co2 fire extinguisher | - Capacity 6 ± 0.15. - Fire rating 60 B. - Working temperature: -20 C° ~ + 55 C°. - Rate pressure test 22.5 Mpa. - Expiry date in best condition: 5 Years. | 2 pcs |  |  |

- WARRANTY & DOCUMENTS:

- A minimum one-year warranty is required.

- GENERAL REMARKS AND SPECIAL CONDITION

- Technical data sheets or catalogues are required for all items

- Pictures are only for understanding the product specifications, not indicative of a specific model or brand.

- All standard accessories for the offered items should be provided.

- The supplier must provide the items with all standard technical features and specifications, in full working condition.

- Packing, loading, transportation, and unloading are the responsibility of the supplier.

- The price should include transportation and installation costs.

- Suppliers can make an offer for one lot or both lots, provided that all items under the lot they offer for are provided.

- All dimensions are approximate, with a tolerance of plus or minus 20 mm.

- All items are subject to thorough checking and approval by the GIZ team and the recipients.

- Delivery Location

- The delivery location is Al-Rumaitha Municipality, allocated to the ONE STOP SHOP space.